OPENFLAME BURNER		GROUP B-3 single jet.

......Spear Point Flame......				Range: SPF.6 to 75 [¼” to 3”]

Pre-Mix.    Air-Gas.

Manufactured from Stainless Steel and designed for operating with a highly stable spearpoint shaped flame with a wide turn-down ratio, this burner nozzle offers excellent resistance when used in a corrosive environment or high temperature atmosphere. Pre-mixing with combustion air is complete with a fan, blower or compressed air supply to the venturi mixer. Mainly used with either Natural or Propane gas this type of nozzle provides an ideal assembly  when combined with the Rapidflame RAM and VM range of Venturi Mixers and control valves.

Typical applications include:

Immersion tube firing......Kilns......Ovens......Preheating lances.

Foundry ladle heating......Pilot burner assemblies......Cremators......Furnaces.

SPF-25M

External thread connection

Cat.		Type.		Thread		Overall		Kcal/Hr.

No.				  size	 	 length		nominal

B030		SPF-6M		¼”    	bsp	  40		1250

B031		SPF-9M		3/8”    	bsp	  72		2500

B032		SPF-12M	½”    	bsp	  76		7500

B033		SPF-19M	¾”    	bsp	  76		15000

B034		SPF-25M	1”       	bsp	  90		22000

B035		SPF-32M	1 ¼” 	bsp	  107		45300

B036		SPF-38M	1 ½” 	bsp	  120		71500

B037		SPF-50M	2”       	bsp	  160		113000

B038		SPF-63M	2 ½” 	bsp	  165		220000

B039		SPF-75M	3”       	bsp	  165		375000

Available in carbon steel to special order.

Internal thread connection

Cat.		Type.		Thread		Overall		Kcal/Hr.

No.				  size		 length		nominal

B040		SPF-6F		¼”    	bsp	  40		1250

B041		SPF-9F		3/8”    	bsp	  72		2500

B042		SPF-12F	½”    	bsp	  76		7500

B043		SPF-19F	¾”    	bsp	  76		15000

B044		SPF-25F	1”       	bsp	  90		22000

B045		SPF-32F	1 ¼” 	bsp	  107		45300

B046		SPF-38F	1 ½” 	bsp	  120		71500

B047		SPF-50F	2”       	bsp	  160		113000

B048		SPF-63F	2 ½” 	bsp	  165		220000

B049		SPF-75F	3”       	bsp	  165		375000

Available in carbon steel to special order.

The Rapidflame policy is one of continuous development and improvement, we therefore reserve the right to change any specification or design without prior notice.