FISHTAIL BURNER			Group F-1 Fishtail Burner.

......Machine Mounting......					Range: Oxy-Gas.    Pre-Mix.

Piloted flame type.

Stainless Steel.

Typical applications include:

Vials and Ampoules on Rotary Index

and Walking Beam Machines.

Shaping and Forming of Lamps and

Lighting components.

Laboratory Glassware manufacturing.

Automated and low volume production

of Tabletop Glassware and Containers.

Steel heat treatment and Metal Joining

processes.

1081                             	    1012

Cat.No.	Type.	No. of	  #	 Kcal/hr	    A	     B	   C	 D	   E	    F             G

Holes	Drill     	nominal

F001	1012	   4	 64	  1575	   6.7	  14.5	  22	16	  6.4	   6.4       1/4X26 tpi

F002	1012-B	   4	 74	  525	   6.7	  14.5	  22	16	  6.4	   6.4       1/4X26 tpi

F003	1012-C	   6	 78	  394	   6.7	  14.5	  22	16	  6.4	   6.4       1/4X26 tpi

F004	1012-E	   5	 78	  370	   5.2	  14.5	  22	16	  6.4	   6.4       1/4X26 tpi

F005	1012-F	   5	 71	  920	   5.2	  14.5	  22	16	  6.4	   6.4       1/4X26 tpi

F006	1012-G	   4	 64	  1706	   5.2	  14.5	  22	16	  6.4	   6.4       1/4X26 tpi

F007	1012-H	   5	 78	  370	   5.2	  14.5	  22	16	  6.4	   6.4       1/4X26 tpi

F008	1012-J	   6	 70	  1312	   5.6	  14.5	  22	16	  6.4	   6.4       1/4X26 tpi

F009	1081	   9	 62	  3000	   15.5	  20.5	  33	20.5	  9.5	   7.9       1/8BSPint.

F010	1081-A	   9	 62	  3000	   15.5	  20.5	  33	20.5	  9.5	   7.9       1/4BSPext.

F011	1081-B	   9	 62	  3200	   13	  20.5	  33	20.5	  9.5	   7.9       1/4BSPext.

F012	1081-C	   9	 74	  3200	   15	  20.5	  33	20.5	  9.5	   7.9       1/4BSPext.

F013	1081-E	   5	 62	  2300	   15.5	  20.5	  33	20.5	  9.5	   7.9       1/4BSPext.

F014	1081-F	   10	 71	  1600	   15.5	  20.5	  33	20.5	  9.5	   7.9       1/4BSPext.

F015	1081-BD     9	 62	  3000	   15.5	  20.5	  33	20.5	  9.5	   7.9    1/8 int. x ¼ ext.

Type 1012 is also available with M6x1 ext. thread and 1/4x27 tpi ext thread.  Type 1081 is also available with NPT ext. thread.

Type	Flame Pattern	Type	Flame Pattern

1012	Divergent		1081	Divergent

1012-B	Divergent		1081-A	Divergent

1012-C	Parallel		1081-B	Parallel

1012-E	Parallel		1081-C	Divergent

1012-f	Parallel		1081-E	Divergent

1012-G	Parallel		1081-F	Divergent

1012-H	Divergent

1012-J	Parallel

To ensure that accurate and constant flame settings

are achieved, all pre-mix Oxy - Gas burners must

operate from a correctly sized  venturi mixer, together

with compatible needle type flow control valves.

If necessary, contact the Rapidflame technical office

for assistance or advice on the selection of either new

or existing venturi mixers.